CST 133

Digital Electronics Design with Verilog

Lab 0 – Simulation tutorial

Laboratory Objective #1: Understand how to use the ModelSim simulator to simulate digital logic circuits.

Laboratory Objective #1:  Define a majority gate.

Laboratory Objective #3: Reinforce truth tables.

- What does it mean when we say ‘majority rules’?

If a majority of the input signals are high – only two out of three in this case – then the output is high as well.

- The majority gate is defined by the following expression.

- Please fill out the truth table below.

| x1 | x2 | x3 | x1x2 | x1x3 | x2x3 | x1x2+x1x3+x2x3 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |
| - | - | - | - | - | - | - |

- Please provide a schematic representation of the gate (draw it out using discrete gates).

- Okay, now follow the ModelSim tutorial here.

You might want to watch this YouTube video of how to use ModelSim before going through the Introduction. https://www.youtube.com/watch?time_continue=220&v=jIihuC9Mxz0

- Provide a screenshot similar to Figure 30.

Submission

Submissions

The instructor does not guarantee last second availability. It is your responsibility to submit the lab in a timely manner. Note that the instructor’s mailbox may not be open after 5 PM.

A penalty of 20% per day up to 4 days will apply. After 4 days, no submissions will be accepted and you will receive a score of 0 for the lab.

For this lab you must submit the following to your instructor’s mailbox by 9 PM Pacific Standard Time on the day before your next scheduled lab.

- This lab document completed in its entirety, with all supporting documents attached.

- ModelSim screenshot that you created similar to Figure 30.